
GSM Dual-Band GPRS Digital
Mobile Phone
User Manual
The manual is applicable for the Rio from Orange mobile phone.
文档信息

	版本
	修订日期
	修订人
	软件版本
	备注

	1.0
	2010.2
	赵琼
	X991_V1_Z2_FRES_D18F101 
	2010.1.30 18:00

	1.0
	2010.3
	赵琼
	X991_V1_Z2_FRES_D18F105
	2010.2.26 18:00

	
	2010.3
	赵琼
	X991_V1_Z2_FRES_D18F106
	2010.3.12 18:00


Copyright © 2010 by ZTE Corporation
All rights reserved.

No part of this publication may be excerpted, reproduced, translated or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without the prior written permission of ZTE Corporation.

The Bluetooth® word mark and logos are owned by the Bluetooth SIG, Inc. and any use of such marks by ZTE Corporation is under license. Other trademarks and trade names are the property of their respective owners.

Java and all Java-based marks are trademarks or registered trademarks of Sun Microsystems, Inc.

ZTE Corporation reserves the right to make modifications on print errors or update specifications in this manual without prior notice.

Version 1.0 in Feb. 2010
Contents
1Before Use


1Safety Instructions


2Precautions for using the mobile phone


6Precautions for using the battery


7Precautions for using the charger


9Parts Definition


12Insert and Remove SIM Card


13Inserting & Removing the Memory Card


14Battery


14Installing the battery


14Removing the battery


15Charging the battery


16Power On/Off


17Using your phone


18Descriptions of common functions


18Widgets


18Making & receiving calls


18Making a call


19Receiving a call


19Adjusting the volume


19Hands-free


20Call options


20Writing messages


21Viewing messages


21Add new contact


22Sending and receiving Email


23Using the phone as a memory storage device


24Main Function list


33Input Methods


34Appendix


34Clearance and Maintenance


36Health and safety information


36SAR certification information


37Health & Safety information


37Limiting exposure to radio frequency (RF) fields


38Declaration of RoHS Compliance


39Disposal of your old device


40EC DECLARATION OF CONFORMITY


42Limited warranty statement


46Failure Analysis





Before Use
Thank you for choosing the Rio from Orange, a GSM Dual-Band GPRS Digital Mobile Phone (referred to as the “mobile phone” in this manual).

To guarantee your mobile phone is always in its best condition, please read this manual carefully and keep it for future reference.

The pictures in this user manual are only used to demonstrate the mobile phone’s functions, so they may be different from what you see on your mobile phone.
Safety Instructions
If your mobile phone is lost or stolen, notify Orange immediately to prevent others from abusing it. To protect your mobile phone against abuse, take the following precautions:

· Set a PIN code and modify it from time to time.
· Keep your mobile phone with you at all times.
Precautions for using the mobile phone
[image: image1.jpg]£
QY



 Road Safety
· Pay attention to road safety and follow all local relevant laws and regulations while driving.

[image: image2.jpg]


 Interference 
· Do not use your mobile phone near sensitive electronic equipment. Radio wave interference may cause electronic equipment to malfunction. Pay special attention when you use your mobile phone near audio-phones, pacemakers and/or other medical electronic equipment, fire detectors, automatic-doors and/or other automatic-control equipment. For details on the influence caused by your mobile phone on pacemakers or other medical electronic equipment, please consult the manufacturer or your local distributor.

· Do not use your mobile phone near TV sets, radios or office automated equipment to avoid interference.

[image: image3.jpg]&%)
&



 On an Aeroplane
· Turn off your mobile phone on an aeroplane when use is forbidden. The mobile phone may affect the normal operation of electronic devices, therefore please follow the relevant regulations when using your mobile phone on an aeroplane.

[image: image4.jpg]&)



 In Hospital
· Turn off your mobile phone in hospital where it’s forbidden to use it. The mobile phone may affect the normal operation of medical apparatus, therefore please follow the relevant regulations when using your mobile phone in hospital.

[image: image5.jpg]O



 At the Petrol Station
· Do not use your mobile phone in a petrol station. Turn off your mobile phone near fuels or chemicals.

[image: image6.jpg]%
D



 Waterproof
· Keep your mobile phone away from water. This may cause overheating, electrical leakage or failure if water or any other kind of liquid enters your mobile phone.

[image: image7.jpg]£
&S



 Disassembly
· Do not disassemble or reassemble your mobile phone as this may cause damage, electrical leakage or circuit failure.

[image: image8.jpg]


 Antenna
· Do not use your mobile phone if your antenna is damaged as this may cause injury.

[image: image9.jpg]


 Away from Children
· Keep your mobile phone from children. Do not allow children to use the mobile phone as a toy as this may be harmful to their health. 

[image: image10.jpg]


 Original Accessories
· Use the original accessories or those authenticated by the manufacturer. Using non-original accessories may affect the mobile phone’s performance, violate the warranty provisions or relevant national regulations on mobile phones, or even cause bodily harm.

[image: image11.jpg]


 Areas where industrial explosives are used
· Please strictly adhere to relevant laws and regulations to turn off your mobile phone in or near areas where explosive materials are used. 

[image: image12.jpg]


 Emergency Calls
· Make sure your mobile phone is turned on and in the service area. Input the national emergency number and press the Send key to dial. Let others know where you are and do not hang up until help has reached you.
Precautions for using the battery
· Keep the battery in a cool and ventilated place, out of direct sunlight.

· Battery life is limited, and becomes shorter and shorter after the battery is charged continuously over its lifetime. If charging fails several times, this indicates that the battery is worn out and a replacement is advised.
· Dispose of worn-out batteries in designated locations according to specified regulations rather than throwing them into residential garbage.
· Do not throw worn-out batteries into fires as this may cause additional fires or explosions.
· Do not press with great force when installing the battery because this may result in leakage, overheating, explosion or fire.
· Do not disassemble or reassemble the battery because this may result in leakage, overheating, explosion or fire.
· If the battery becomes hot, changes colour or deforms during use, charging or storage, please immediately stop using the device and replace the battery.
· Keep the battery away from dampness to prevent overheating, smouldering or erosion.
· Do not leave the battery in direct sunlight, for example in a car, or other high-temperature location as this may result in leakage or overheating. This may also affect the battery’s performance and shorten its life.
· Do not continuously charge the battery over a 24 hour period.  
[image: image13.png]



Note:
If the battery is damaged, please avoid contact with the materials inside. If your skin has contact with these materials, flush your skin with lots of water and seek medical assistance if required.
Precautions for using the charger
· Use power voltage between 110VAC and 220VAC. Using voltage beyond the range may cause leakage, fire or damage to the phone and charger.
· Do not short-circuit the charger because this may cause electric shock, smouldering or damage.
· Do not charge the battery in direct sunslight, near dampness, dust, vibration, TV set, or radio (may affect the image, sound), etc.
· Do not use the charger if the power cable is damaged because this may cause fire or electric shock.
· Do not place water containers near the charger to avoid water splashing. If water or any other kind of liquid splashes on the charger, immediately remove the charger from the power socket to avoid overheating, fire, electric shock or failure.
· Do not disassemble or reassemble the charger because this may cause physical injury, electric shock, fire or damage.

· To avoid electric shock, do not touch any charger, power cable or power socket with wet hands.
· Do not place heavy objects onto power cables or refit power cable as this may cause electric shock or fire.
· Remove the power plug from the socket before cleaning and maintenance.
· Hold the charger rather than pull the power cable while removing the plug from the socket as this may cause damage, electric shock or fire.
Parts Definition
[image: image197.png]




[image: image14]

[image: image15]
[image: image16.png]



Note:

In this user manual, “press” means pressing the key and then releasing; “hold” means pressing and holding the key for about 2 seconds or more.
Descriptions of icons on the status bar 
	Icons
	Descriptions
	Icons
	Descriptions

	
[image: image17.png]



	Show the phone’s network signal. strength.
	
[image: image18.png]



	Show the phone’s battery capacity.

	[image: image19.jpg]B




	Calling or in call
	[image: image20.jpg]



	You have missed calls.

	[image: image21.jpg]By




	Call Holding
	
[image: image22.png]



	Call Forwarding

	[image: image23.jpg]



	You have unread SMS.
	[image: image24.jpg]



	You have unheard voice mail.

	[image: image25.jpg]



	You have unread MMS.
	[image: image26.jpg]@




	You have unread email.

	[image: image27.jpg]



	The ringing type is set to Ring.
	[image: image28.jpg]



	The ringing type is set to Vibration. 

	
[image: image29.png]



	The ringing type is set to vibration and ring tone.
	[image: image30.jpg]



	The ringing type is set to Mute.

	[image: image31.jpg]



	Audio player on
	[image: image32.jpg]



	Bluetooth® ON

	[image: image33.jpg]



	A headset is connected to the phone.
	[image: image34.jpg]



	The phone is under roaming status.

	[image: image35.jpg]



	A GPRS packet data connection is active.
	[image: image36.jpg]



	A EDGE packet data connection is active.

	[image: image37.jpg]



	A stopwatch is active.
	[image: image38.jpg]



	A clock alarm is active.

	[image: image39.jpg]



	The keypad is locked.
	[image: image40.jpg]



	Keypad unlocked.


Insert and Remove SIM Card
[image: image198.png]


Insert a valid SIM card into your mobile phone before use. All information regarding network connection and the SIM phone book is saved on the SIM card. In order to avoid loss and damage of saved information on the SIM card, please do not touch the metal surface and store the SIM card far away from electricity or a magnetic source. 
1. Turn off your mobile phone, remove the battery and disconnect the external power.
2. Insert the SIM card into the socket.
3. Turn off your mobile phone and remove the battery before taking out the SIM card. 
Inserting & Removing the Memory Card
[image: image41.png]



Note:

Please do not take out the memory card when the card is in use. Taking out the card may not only damage your card and mobile phone, but also destroy the data saved on the card.
1. Open the memory card cover at the right side of the phone and insert the memory card.
2. [image: image199.jpg]


Push tightly until the card is installed in the right position and close the card cover.
3. Before you take out the card, please open the card cover and press the card inward to unlock.
[image: image42.jpg]


     [image: image43.jpg]



Battery
Installing the battery
	1. Align the battery's contact points in the direction of the arrow as shown, and Insert the battery into the slot.
2. Put on the battery cover, and push forward in the direction of the arrow as shown until it’s locked.
	[image: image44.jpg]


[image: image45.jpg]





Removing the battery
	1. Turn off the mobile phone.
2. Push the battery cover in the direction of the arrow as shown and open the cover. 
3. Pull the battery in the direction of the arrow as shown and take out the battery. 
	[image: image46.jpg]


[image: image47.jpg]





Charging the battery
[image: image200.png]


The accompanying Li battery can be used after being unpacked. To ensure the battery is always in its best condition, please exhaust the capacity of the battery and then fully recharge it for the first three times of usage.
1. Connect the charger with the mobile phone through the charging interface, and insert the charger to a 110V/220V power socket.
2. When the phone is powered on, the capacity indicator on top-right corner of the screen will scroll continuously during charging; when the phone is powered off, there’ll be charging indication on the screen during charging. If the phone is used excessively without enough capacity, the charging indicator will not immediately appear on the screen of the phone when charging starts, and the phone may not be powered on for a few minutes. 
3. When the phone is on, the battery will be fully charged when the capacity indicator on the up-right corner of the screen is full and no longer scrolls. When the phone is off, the battery will be fully charged when there is a completed charging indication on the screen. During charging, the phone and charger may normally become hot.
4. Please disconnect the charger from your mobile phone and the socket when the charging is completed.
Power On/Off
Hold the Power key until the power-on/off animation appears.
After you turn on your mobile phone, it will automatically check whether or not a SIM card is inserted or usable, and may show the following information on the screen, if you have set a lock code or a PIN code:
· Input phone lock code
· Input SIM PIN
[image: image48.png]



Note:
The SIM card will be locked if you enter an incorrect PIN code three times. If this occurs, you will need a PUK code to unlock the SIM card. The PIN code may be provided to you together with SIM card. If not, contact your network service provider. The default PIN code is normally 1234. Please replace it with your own PIN code as soon as possible.
· Searching: The mobile phone will search for an available network. 
Using your phone
	Call history
Contacts
Messenger

Orange Widgets
Organiser
Calendar

Alarm

Tasks

Calculator

File manager

Messaging

Email

Write SMS&MMS

Inbox

Drafts

Outbox

Sent messages

Archive

 
	Delete messages

Broadcast messages

Templates

Message settings

Text message counter

Orange World

Music

Orange store

Audio player

Games&applications
Java Game&Apps

Other Games

Stopwatch 

Unit converter

Currency converter

World clock
Multimedia

Camera 
Gallery


	Video recorder
Video player
Sound recorder

FM radio

Sound effects

Settings
My profiles

Bluetooth®
Phone settings
Call settings

Network settings

Security settings
Java settings

Java default security

Screen calibration

Memory settings

Restore factory settings


	More

STK

Orange World

Opera Mini

Data account

Licence manager

Conn.management

Data counter




Descriptions of common functions
Widgets
In the idle screen, slide your finger to the left or right on the screen to switch among the three interfaces.
In idle mode, click [image: image49.jpg]


 to enter the Widgets interface. In the Widget drawer, slide your finger up and down in the area to display other icons. You can drag the icons from Widget drawer to the idle screen.
Click [image: image50.png]


 to enter the Shortcut area. Slide your finger on the screen to the left or right to display other icons.
Making & receiving calls
Making a call 
· Direct dial: In idle mode, press the numeric keys to input the telephone number and press the Send key to dial. 
· Dialling from the Contacts: In idle mode, press right soft key to enter “Contacts”, select the contact and press the Send key to dial. 
· Dialling from call logs: In idle mode, press Send key to enter the list of call logs, select the call record and press Send key to dial. 
· Dialling an extension: Sometimes you can not dial the extension directly. Dial the direct line firstly, press “P” key and then dial the extension number. 
· Dialling international calls: To dial an international call, press “+” key and then dial the country or region code, city code and phone number.
Receiving a call
You can press the Send key to answer the call.
Adjusting the volume
During the conversation, press the Side keys to adjust the voice volume; In idle mode, press the Side keys to adjust the key volume.

Hands-free
During the conversation, press the “H-Free” key to enter the hands-free status, and press “H-Held” to return to normal status. 
Call options
During the conversation, press “Options” to enter “Hold single call”, “New call”, “Messaging”, etc. 
Writing messages
1. In idle mode, press the OK key to enter Menu, then select “Messaging”[image: image51.png]


“Write SMS&MMS” to edit the message text.
2. Write your message:
· If you want to add a picture, sound, video etc., please press “Options” to select “Add picture”, “Add sound” or “Add video” etc. 

· If you want to send a multimedia message, please press OK key to select “Switch to MMS”.

3. After editing the message, press “Options” to select “Send to”. 
4. Select: 
· Enter recipient: Manually input the recipient’s number. 

· Add from Contact: Select the recipient from Contacts. 
5. Select “Send” to send the message. 
Viewing messages
Upon receiving a message, the mobile phone can indicate with a ring tone, prompt, or a new message icon.

Press the OK key to enter the Inbox, and then read the message.
1. In idle mode, press the OK key to enter main menu, then select “Messaging”[image: image52.png]


“Inbox”.
2. The list of received messages will be displayed on the screen. Press the Up/down key to select the message, and press OK key to view the contents. 
3. Press “Options” to select “Reply”, “Call sender”, “Forward”, etc. 
Add new contact
1. In idle mode, press the OK key to enter the main menu and select the “Contacts”.
2. If you want to add a new entry to phone, please select “Add new contact”. You can edit “Name”, “Number”, “Home Number”, etc.
3. If you want to add a new entry onto the SIM card, select “OK”[image: image53.png]


“Contact settings”[image: image54.png]


“Preferred storage”[image: image55.png]


“SIM”. You need input “Name” and “Number” only.
4. Select “Save” to save after editing.
Sending and receiving Email

You can send and receive Emails over the GPRS network after the mobile phone has connected to the mail server. You have to change the following settings before using Emails. For different email providers, the settings may be slightly different. Please consult your email provider. The following guide is for your reference only.
1. In idle mode, press the OK key and then click “Messaging” to select “Email”[image: image56.png]


“Email accounts” to create a new account.
2. Select “Add”[image: image57.png]


“Email setup wizard”, input your Email’s “E-mail address”, “User name”, “Password”.
3. Return to “Email accounts”, select the newly created account, and press “Options”[image: image58.png]


“Activate”.
4. After settings, you should be able to send/receive mails.
Using the phone as a memory storage device
When the phone is turned off, connect your mobile phone to the PC with the USB data cable. Turn on your mobile phone connected to the PC with the USB data cable. If the memory card is inserted, you can select “Mass storage” on the PC, and use the phone as a memory storage device.
Main Function list
In idle mode, press OK key to enter the main menu. 
	Functions
	Descriptions 
	Operations

	View call history
	View the recent incoming, missed and outgoing calls.
	Call history[image: image59.png]


All calls / Dialled calls / Received calls/ Missed calls 

	Delete a call log
	Delete a call log on the SIM.
	Call history[image: image60.png]


Select one call log [image: image61.png]


Options[image: image62.png]


Delete

	View call duration
	View the duration of last call, dialled calls and received calls.
	Call history[image: image63.png]


Options[image: image64.png]


 Advanced[image: image65.png]


Call time

	View contacts 
	Browse all contacts in Contact, input the first letter of contact’s name to quickly search for the contact.
	Contacts[image: image66.png]


View contacts among All contacts, SIM contacts, Group contacts and Phone contacts.

	Add contacts
	Save the telephone number and name.
	Contacts[image: image67.png]


Add new contact

	Edit contact
	Edit the information of existing contact.
	Contacts[image: image68.png]


Select one contact [image: image69.png]


Options[image: image70.png]


Edit

	Copy contacts
	Copy all contacts from the SIM card to phone or from phone to the SIM card. 
	Contacts[image: image71.png]


Move the highlight to “Add new contact” [image: image72.png]


Options[image: image73.png]


Contact Settings[image: image74.png]


Copy contacts

	Move contacts
	Move all contacts from SIM card to phone or from phone to SIM card.
	Contacts[image: image75.png]


Move the highlight to “Add new contact”[image: image76.png]


Options [image: image77.png]


Contact Settings[image: image78.png]


Move contacts

	Delete all contacts
	Delete all contacts in SIM card or phone.
	Contacts[image: image79.png]


Move the highlight to “Add new contact”[image: image80.png]


Options [image: image81.png]


Contact Settings[image: image82.png]


Delete contacts

	Preferred storage
	Select the preferred storage as Phone or SIM.
	Contacts[image: image83.png]


Move the highlight to “Add new contact”[image: image84.png]


Options [image: image85.png]


Contact settings[image: image86.png]


Preferred storage

	Speed dial 
	Edit 8 phone numbers and assign these numbers to the keys from 2 to 9. 

In idle mode, hold the numeric key to quickly dial the corresponding phone number of the numeric key.
	Contacts[image: image87.png]


Move the highlight to “Add new contact”[image: image88.png]


Options[image: image89.png]


 Contact settings[image: image90.png]


Speed dial

	Caller groups 
	Divide the contacts into different groups. You could add/delete the members only when they are saved in Phone.
	Contacts[image: image91.png]


Caller groups

	Messenger
	Log on to Orange Messenger.
	Messenger

	Orange Widgets
	Log on to Orange Widgets.
	Orange Widgets

	Calendar
	Check the calendar and schedule.
	Organiser[image: image92.png]


Calendar

	Alarm
	Set multiple alarms, and support power-off alarm.
	Organiser[image: image93.png]


Alarm

	Tasks
	Add Date, Call, Anniversary, etc. 
	Organiser[image: image94.png]


Tasks

	Calculator
	Simple calculation, such as add, subtract, multiply, divide, etc. 
	Organiser[image: image95.png]


Calculator

	File manager
	View the files saved on the phone and memory card. 
	Organiser[image: image96.png]


File manager

	Email
	Send and receive E-mails. 
	Messaging[image: image97.png]


Email

	Send message
	Send text messages/ multimedia messages to other mobile phones.
	Messaging[image: image98.png]


Write SMS&MMS 

	View message
	Read the messages (received, unsent, sent, drafts, saved).
	Messaging[image: image99.png]


Inbox/ Drafts /Outbox /Sent messages /Archive

	Delete message
	Delete the messages in Inbox / Outbox / Sent message / Drafts / Archive or all messages.
	Messaging[image: image100.png]


Delete messages

	Templates
	Select the template to send text message or multimedia message.
	Messaging[image: image101.png]


Templates

	Voice mail*
	Lets the calling party leave a message through voicemail, and check the voicemail to listen to the message.
	Messaging[image: image102.png]


Message settings[image: image103.png]


Text message[image: image104.png]


 [image: image105.png]


Voicemail server

In idle mode, hold “1” key to call the first voice mail number.

	Check memory status
	Check the memory status for text messages on the SIM card and Phone.
	Messaging[image: image106.png]


Message settings[image: image107.png]


Text message [image: image108.png]


Memory status

	Preferred storage
	Save messages on the SIM card or phone. 
	Messaging[image: image109.png]


Message settings[image: image110.png]


Text message[image: image111.png]


 Preferred storage

	Broadcast messages*
	Receive the messages from network operator.
	Messaging[image: image112.png]


Broadcast messages

	Orange World
	Log on to Orange World.
	Orange World

	Orange store
	Log on to Orange Store.
	Music[image: image113.png]


Orange store

	Audio player
	Play the audio file.
	Music[image: image114.png]


Audio player

	Java Game&Apps
	Get Access to Java games and applications.
	1. Select the Java application..

2. Select "Launch" to enter java application.

	Other Games
	The phone provides preset games. The detailed operation descriptions are in the games menu. 
	Games&application[image: image115.png]


Other Games

	Stopwatch
	Measure the time.
	Games&application[image: image116.png]


Stopwatch

	Unit converter
	Perform conversions of basic units, such as weight, length, etc.
	Games&application[image: image117.png]


Unit converter

	Currency converter
	Set the exchange rate and account foreign currency or vice versa.
	Games&application [image: image118.png]


Currency converter

	World clock
	Browse the current time of different cities around the world.
	Games&application[image: image119.png]


World clock

	Camera
	Capture digital photos.
	Multimedia[image: image120.png]


Camera

	Gallery
	Browse the photos saved on the phone and memory card.
	Multimedia[image: image121.png]


Gallery

	Video recorder
	Record video files.
	Multimedia[image: image122.png]


Video recorder

	Video player
	Play the video file.
	Multimedia[image: image123.png]


Video player

	Sound recorder
	Record audio files.
	Multimedia[image: image124.png]


Sound recorder

	FM radio
	Listen to FM broadcasts.
	Multimedia[image: image125.png]


FM radio

	My profiles
	Perform personalized settings on ringtone, volume, ringer type, etc. according to different events and occasions. 
	Settings[image: image126.png]


My profiles[image: image127.png]


Select Profile (General/Meeting etc.)[image: image128.png]


 Activate
In idle mode, hold 
[image: image129.png]


 key to turn on or off “Meeting” mode. 

	Bluetooth®
	Exchange data with your phone & other Bluetooth® devices, and listen to music or talk hands-free with Bluetooth® headset.
	Settings[image: image130.png]


Bluetooth®

	Date and Time
	Set the date and time displayed on the phone.
	Settings[image: image131.png]


Phone settings[image: image132.png]



Date and time 

	Key vibrate
	Turn on/off key vibrate
	Settings[image: image133.png]


Phone settings[image: image134.png]



Key vibrate

	Sched. power on/off
	Perform the action of power on/off according to the setting time.
	Settings[image: image135.png]


Phone settings[image: image136.png]


 Sched. power on/off

	Language
	Select the language for the menu.
	Settings[image: image137.png]


Phone settings [image: image138.png]


Language

	Dedicated key
	Set the shortcuts for navigation keys in idle mode.
	Settings[image: image139.png]


Phone settings [image: image140.png]


 Dedicated key

	Auto update of date and time
	Select whether or not to update date and time automatically. 
	Settings[image: image141.png]


Phone settings[image: image142.png]


Auto update of date and time

	LCD backlight
	Set the backlight time and brightness.
	Settings[image: image143.png]


Phone settings[image: image144.png]


Misc. settings[image: image145.png]


LCD backlight

	Call waiting
	You will be prompted by the network upon an incoming call if this function is activated. 
	Settings[image: image146.png]


Call settings[image: image147.png]



Call waiting

	Call divert
	Forward the incoming call to designed number on a specific occasion.
	Settings[image: image148.png]


Call settings[image: image149.png]



Call divert

	Call barring
	Set the outgoing call restriction or incoming call restriction.
	Settings[image: image150.png]


Call Settings[image: image151.png]



Call barring 

	Blacklist
	You could edit the Black List. After you select “On” under “Mode”, the incoming call from the numbers in the Black List will be rejected.
	Settings[image: image152.png]


Call Settings [image: image153.png]


More[image: image154.png]


Blacklist

	Auto redial
	When selecting “On” for Auto redial, your phone will automatically redial after a while if the call is not answered.
	Settings[image: image155.png]


Call Settings [image: image156.png]


More[image: image157.png]


Auto redial

	Network settings
	Select the network and set GPRS connection. 
	Settings[image: image158.png]


Network settings

	SIM lock
	Activate PIN code to prevent SIM card abuse.

The default PIN code is normally 1234. Please replace it with your own PIN code as soon as possible.
	Settings[image: image159.png]


Security settings [image: image160.png]


SIM lock

	Phone lock
	Activate phone lock to prevent the phone from abuse.

The default phone lock password is 1122.
	Settings[image: image161.png]


Security settings [image: image162.png]


Phone lock

	Keypad lock 
	The keypad lock would prevent the inadvertent operation caused by key press. 
	Settings[image: image163.png]


Security settings [image: image164.png]


Auto keypad lock

In idle mode, Press [image: image165.jpg]


 key to lock the keypad. Press OK key and [image: image166.jpg]


 key to unlock.

	Fixed dial 
	After this function is activated, you could only dial the number in fixed dial list. 
	Settings[image: image167.png]


Security settings[image: image168.png]


 Fixed dial

	Screen calibration
	Calibrate the screen to click more accurately.
	Settings[image: image169.png]


Screen calibration

	Restore factory settings
	Restore the relevant parameters to factory settings.
	Settings[image: image170.png]


Restore factory settings 

	Data counter
	View the Data counter.
	More[image: image171.png]


Data counter


[image: image172.png]



Note:

When copying files from PC to the Memory Card, please put the files in the relevant folder, e.g., put the music files in the “My Music” folder, and video files into the “Videos” folder.
Input Methods
The mobile phone supports ABC/abc input method, 123 input method and symbol input method.
	Switch input method 
	Press [image: image173.jpg]


 key

	Input space 
	Press 
[image: image174.png]


 key.

	Delete inputs
	Press [image: image175.jpg]


 key to delete the input before the cursor and hold “Clear” to delete all inputs. 

	Switch caps lock 
	Press 
[image: image176.jpg]


 key.

	Line feed
	Press [image: image177.png]


 key.

	Input letters and common symbols 
	There are two symbols on the keys. In ABC /abc method, press the key to input the symbol located on the lower part of the key; press [image: image178.png]


 key and then press the key to input the symbol located on the upper part of the key; press [image: image179.png]


 key twice and then press the key to continuously input the symbol located on the upper part of the key.

	Input digits
	In 123 method, press the numeric keys to input the corresponding numbers.. 

	Input symbol
	During text input, press 
[image: image180.png]


 key to display symbol lists, press navigation keys to select the symbol, and press “OK” confirm. 


Appendix
Clearance and Maintenance
Please carefully maintain your mobile phone as it is a delicately designed and manufactured product. The following advice may help you use your mobile phone correctly and prolong the mobile phone’s life.
· Clean the phone, battery and charger with a soft dry cloth.
· Do not clean the phone with liquids such as alcohol, dilution agent or benzene.
· Clean the socket from time to time as a dusty socket may cause disconnection, power failure, or even charging failure.
· Do not press the keys or screen with needles, pen points or other sharp objects to avoid damage or malfunction.
· Do not touch the mobile phone with wet hands as this may cause an electric shock, which could cause injury or damage the phone.
· Do not use or put your mobile phone in a dusty or dirty place as it may damage the components of the phone.
· Do not put your mobile phone in a location of too high/low temperature.
· Do not put your mobile phone in or on heating devices (microwave, ovens or heat sinks). The mobile phone may explode if it gets too hot.
· The colour of the waterproof label in the phone will change if the phone or battery gets wet. In this case, the warranty provisions will become void even if the warranty period has not expired.
· If there is anything wrong with the mobile phone, battery, charger or any phone accessory, please send them to your nearest service centre for inspection. 
Health and safety information

SAR certification information

THIS MODEL MEETS INTERNATIONAL GUIDELINES FOR EXPOSURE TO RADIO WAVES.
Your mobile phone is a radio transmitter and receiver. It is designed not to exceed the limits for exposure to radio waves recommended by international guidelines. The guidelines were developed by an independent technical standards body, the ICNIRP, and include margins designed to assure the protection of all persons, regardless of age and health.

These guidelines use a unit of measurement known as Specific Absorption Rate, or SAR. The SAR limit for mobile devices is 2.0 W/kg and the highest SAR value for this device tested was1 :
	Position
	GSM900MHz(W/Kg)
	GSM1800MHz(W/Kg)

	Head
	0.483
	0.295

	Body
	0.262
	0.200


As SAR is measured utilizing the device’s highest transmitting power, the actual SAR of this device while operating is typically below that indicated above. This is due to automatic changes to the power level of the device to ensure it only uses the minimum level required to reach the network.

The World Health Organization has stated that present scientific information does not indicate the need for any special precautions for the use of mobile devices.
[image: image201.png]


1. The tests are carried out in accordance with IEC standard PT62209-1.
Health & Safety information
Important safety information includes the information about radio frequency (RF) radiation exposure. To ensure compliance with RF exposure guidelines the phone must be used at a minimum of 1.5cm away from the body.

Failure to follow these instructions could cause the RF exposure to exceed the relevant guideline limits.
Limiting exposure to radio frequency (RF) fields

For individuals concerned about limiting their exposure to RF fields, the World Health Organization (WHO) provides the following advice:

Precautionary measures: Present scientific information does not indicate the need for any special precautions for the use of mobile phones. If individuals are concerned, they may choose to limit their own or their children’s RF exposure by limiting the length of calls, or using ‘hands-free’ devices to keep mobile phones away from the head and body.

Further information on this subject can be obtained at the homepage of the World Health Organization: http://www.who.int/peh-emf WHO Fact sheet 193: June 2000.
Declaration of RoHS Compliance

To minimize the environmental impact and take more responsibility for the

earth we live on, this document shall serve as formal declaration that the Rio from Orange manufactured by ZTE CORPORATION is in compliance with the Directive 2002/95/EC of the European Parliament - RoHS (Restriction of Hazardous Substances) with respect to the following substances:

(1) Lead (Pb)

(2) Mercury (Hg)

(3) Cadmium (Cd)

(4) Hexavalent Chromium (Cr (VI))

(5) Polybrominated biphenyls (PBB’s)

(6) Polybrominated diphenyl ethers (PBDE’s)

(Compliance is evidenced by written declaration from our suppliers, assuring that any potential trace contamination levels of the substances listed above are below the maximum level set by EU 2002/95/EC, or are exempt due to their application.)

The Rio from Orange manufactured by ZTE CORPORATION, meets the requirements of EU 2002/95/EC.
Disposal of your old device
[image: image202.png]


1. When this crossed-out wheeled bin symbol is attached to a product it means the product is covered by the European Directive 2002/96/CE.

2. All electrical and electronic products should be disposed of separately from the municipal waste stream via designated collection facilities appointed by the government or the local authorities.

3. The correct disposal of your old appliance will help prevent potential negative consequences for the environment and human health.

EC DECLARATION OF CONFORMITY [image: image181.jpg]



It is hereby declared that following designated product: 

Product Type: GSM Dual-band Digital Mobile Phone 

Model No.: Rio from Orange (ZTE-G X991)
complies with the essential protection requirements of Low Voltage Directive 2006/95/EC (previous 73/23/EEC), Electromagnetic Compatibility Directive 2004/108/EC (previous 89/336/EEC), Radio and Telecommunication Terminal Equipment Directive (EC Directive 1999/05/EC) and their amendments.

This declaration applies to all specimens manufactured identical to the samples submitted for testing/evaluation.

Assessment of compliance of the product with the requirements relating to Low Voltage Directive 2006/95/EC (previous 73/23/EEC), Electromagnetic Compatibility Directive 2004/108/EC (previous 89/336/EEC), Radio and Telecommunication Terminal Equipment Directive 1999/05/EC and their amendments was performed by Phoenix Testlab (Notified Body No. 0700), based on the following standards:

	Essential Requirement
	Applied  Specifications/Standards

	Art.3.1(a)
	Health
	EN50360, EN50361

	Art.3.1(a)
	Safety
	EN60950-1

	Art.3.1(b)
	EMC
	EN301489-1, EN301489-7

	Art.3.2
	Radio
	EN301511


This declaration is the responsibility of the manufacturer: 

ZTE Corporation


ZTE Plaza, Keji Road South, Hi-Tech, Industrial Park,


Nanshan District, Shenzhen, Guangdong, 518057, P.R. China

[image: image203.png]



Authorised person signing for the company:

Xu feng     Quality Director Of ZTE Corporation  

Name in block letters & position in the company 

Shenzhen, 23 Dec. 2009         [image: image182.png]5 ¥z




Place & date               Legally valid signature
Limited warranty statement
Article 1

The guarantee for the Orange device (continued as: Device) amounts to 24 (twenty four) months from the date the Device was bought.
Six (6) months for the following consumable parts and accessories batteries, chargers, desk stands, headsets, cables and covers.

The manufacturer guarantees that the Device will properly function with proper usage and maintenance according to the instructions of the manufacturer. The guarantee is applied for all Device faults which came to be because of eventual errors in the material and/or the manufacturing of the Device. The guarantee can be used against presentation of the verified guarantee certificate and invoice. The guarantee does not affect other rights belonging to the buyer on other legal bases. The guarantee is valid in the territory of the U.K. 
Article 2

The guarantee does not cover: faults occurring because of unauthorized modifications of the software, mechanical damages, improper handling, and higher forces (lightning, fire, unsuitable voltage and polarities, traffic accidents, humidity, water and alike), batteries and consumable material including*:

a) Damages caused by using the Device outside of the instructions of the manufacturer and other documentation which is enclosed with the Device as well as damages caused by using with other products of another non-approved manufacturer;

b) Damages caused by accidents, improper and wrong usage, floods, fire, earthquakes and superficial damages which include but are not limited to scratches, dents, broken plastic and alike;

c) Damages caused by servicing (including upgrading and extension) by an unauthorized person as well as a fault of the Device or any part of it if they are altered because of the change of the functionality or other properties without prior written authorization of the manufacturer;

d) Consumable parts other than in the case of damage brought on because of faults in the material or manufacturing;

e) Impossibility of partial and/or complete usage of the Device because of unauthorized modification of the software.

Article 3

Orange is obliged during the guarantee, in a reasonable time period, to remove all reported faults and defects of the Device covered with this guarantee certificate which have come to be with the proper and regular use of the Device, to repair the Device in an authorized service or a replacement of the faulty Device with the same but properly working Device if a repair is not possible.
Part(s) repaired or replacement product during the Warranty Period will be warranted for the remainder of the original Warranty Period or for sixty (60) days from the date of repair or replacement, whichever is longer.
Article 4

The guarantee stops being valid if:

- the information from the invoice and/or the guarantee are erased or altered;

- the information on the Device (type, and serial number of the manufacturer) are erased or altered;

- it is established that the Device was opened by an unauthorized person.

Note: customers can report faults and establish the guarantee with the salesman or authorized services.
* Concrete cases of faults or damages which are not covered by the guarantee include:

· Damages caused by intentional separation, opening or disassembling the enclosure;

· Damages or faults with the operation caused by the removal and insertion of exterior and interior parts of the Device;

· Damages caused by holes in the enclosure;

· Cracked glass or LCD because of outside factors;

· Damages or faults caused by an outside force;

· Damages or faults caused by liquids;

· Damages or faults caused by liquid or dry chemical substances (such as gasoline, acids, corrosive material), dust, dirt and other foreign objects;

· Damages or faults caused by outside heat or fire (damage by interior heat is covered if it did not occur through an outside reason);

· Superficial damages occurring through external reasons, including dents, shabbiness, scratches on the enclosure, discoloration, cracks, peeling or metal or plastic parts of the enclosure.

** The customer is advised to regularly make copies of his information and other contents on the Device and to erase the same from the memory of the Device before handing the Device over to the service. In case the customer does not do this, Orange is not responsible for the information in the memory in the Device to the customer or a third party.
Failure Analysis
	Symptoms
	Possible Causes
	Solutions

	Unable to turn on the phone
	Inadequate battery capacity
	Charge the battery

	
	Poor contact
	Reinstall the battery

	Automatic

power-off
	Seriously poor battery capacity
	Recharge the battery

	Inadequate standby time
	Deceasing of battery performance 
	Replace the battery

	
	Poor network signal
	Turn off the phone as the signal is poor.

	Unable to connect the network 
	Poor network signal
	Move to other places

	
	Out of the service area
	Make sure your phone is in network service area.

	Call drop
	Poor network signal
	Move to other places

	Do not ring upon incoming calls
	Set “Mute” or minimum volume
	Enter “My Profiles” to set Non-silent or check the ringing volume.

	Prompt to input PUK
	SIM card locked due to the continuous entering of wrong PIN code for three times.
	Contact your operator.


[image: image183.bmp][image: image184.bmp][image: image185.bmp][image: image186.bmp][image: image187.jpg]


[image: image188.jpg]


[image: image189.jpg]


[image: image190.jpg]


[image: image191.jpg]


[image: image192.jpg]


[image: image193.jpg]


[image: image194.jpg]B



[image: image195.jpg]


[image: image196.jpg]@




Space Key





Navigation keys and OK key





Left/right soft key











End key








Receiver





Delete key





Send key





Carries out the operations at the left-bottom/right-bottom corner of the screen.





Dial a number or receive incoming calls; In idle mode, press the key to view all recent call.





Return key





Fn key


Switch the input of the symbols located on the upper part and lower part on the key.





Caps lock key





Lock key


Press it to switch the input method, and hold it to quickly lock the keypad.




















SYM key


When editing the text, press it to switch to the symbol input method; 


In idle mode, hold it to quickly switch to Meeting Mode or other mode.








Camera shortcut key








In idle mode:


Up key: Write SMS&MMS


Down key: Inbox


Left/Right key: Switch Interface


OK key: Enter main menu
































Ok key





Internet shortcut key





Power key


Hold this key to turn on or off the phone.











Camera head














USB interface





Self portrait mirror





Side keys


Adjust the volume








Hanging hole





Speaker





Headphone jack





Memory card socket












_1330344800

_1330352910

_1330351734

_1330344767

_1327303157

_1327303270

_1327302714

